Assignment 1

IoT Systems • Hands-On #3

| Student | Itzhak Davidov |
| --- | --- |
| ID - last 4 digits | 6233 |
| Broker | broker.hivemq.com : 1883 |
| Submission date | 14 September 2026 |

Publisher + Subscriber implementation, Tests 1–5, Retained Message, Last Will and 90-second Keep Alive

# 1. Objective

The goal of this assignment is to implement MQTT publisher and subscriber clients, configure session persistence and Quality of Service (QoS), and observe how these settings affect message delivery. The solution uses a unique client/topic namespace based on the last four ID digits: 6233.

# 2. Assignment requirements and implementation status

| Requirement | Status | Implementation |
| --- | --- | --- |
| Own client ID / topic ending with 6233 | Implemented | IoT_ID_6233_* and iot/home_ID6233/..._6233 |
| HiveMQ broker, port 1883 | Implemented | broker.hivemq.com : 1883 |
| Clean Session + QoS Tests 1–5 | Implemented | Selectable configurations and automated test runner |
| Retained Message | Implemented | --retained-demo publishes with retain=True |
| Last Will Message | Implemented | client.will_set(..., qos=1, retain=True) |
| Keep Alive = 90 sec | Implemented | KEEPALIVE = 90 |
| Monitor publish/subscribe results | Implemented | Console output + live broker screenshots (5/5 tests passed) |

# 3. MQTT configuration

| Parameter | Value |
| --- | --- |
| Broker | broker.hivemq.com |
| Port | 1883 |
| Keep Alive | 90 seconds |
| Publish topic | iot/home_ID6233/sensor_6233 |
| Retained topic | iot/home_ID6233/retained_6233 |
| Last Will topic | iot/home_ID6233/status_6233 |
| Publisher Client ID | IoT_ID_6233_PUB_T<test> |
| Subscriber Client ID | IoT_ID_6233_SUB_T<test> |

Figure 1 Publisher configuration with unique 6233 namespace, broker, port and 90-second keep-alive.

# 4. Publisher implementation

The publisher selects the required test configuration, connects using MQTT v3.1.1, defines a Last Will message, and publishes either a normal or retained message. QoS is selected from the Test 1–5 mapping. The retained-message demo uses a dedicated topic and the MQTT retain flag.

Figure 2 Publisher: Last Will definition and publish logic.

# 5. Subscriber implementation

The subscriber uses the same test mapping. It subscribes with the required QoS and prints the topic, payload, received QoS and retained flag for each message. For persistent-session verification, --skip-subscribe reconnects the same client ID without issuing a new SUBSCRIBE request.

Figure 3 Subscriber: subscription and message-receive logic.

# 6. Test procedure

For each test, the subscriber first establishes the subscription and then disconnects. A publisher sends a message while the subscriber is offline. The same subscriber client ID reconnects without re-subscribing. This isolates whether the broker stores the session and queues the offline publication.

# 7. Tests 1-5 results

| Test | Clean Session | Sub QoS | Pub QoS | Expected offline delivery | Protocol result | Status |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | True | 0 | 0 | NO | NO | PASS |
| 2 | False | 0 | 0 | NO | NO | PASS |
| 3 | True | 1 | 1 | NO | NO | PASS |
| 4 | False | 1 | 1 | YES | YES | PASS |
| 5 | False | 1 | 0 | NO | NO | PASS |

Interpretation: offline queued delivery requires a persistent session (clean_session=False) and an effective delivery QoS above 0. Effective QoS is limited by the lower of publish QoS and subscription QoS. Therefore Test 4 is the only one of Tests 1–5 that queues the offline message.

Test 2 note: the supplied slide contains wording that can be read as “messages to be received”, but its own note correctly states that QoS 1 or above is also required. With QoS 0, an offline publication is not queued for later delivery.

## 8. Explanation by test

Test 1 Clean session + QoS 0: session/subscription state is not kept, and QoS 0 messages are not queued. Result: no offline message after reconnect.

Test 2 Persistent session + QoS 0: the subscription can be remembered, but the offline QoS 0 publication is not stored. Result: no offline message after reconnect.

Test 3 Clean session + QoS 1: QoS is reliable while connected, but the clean session removes persistent subscription/session state. Result: no offline message after reconnect.

Test 4 Persistent session + QoS 1: the broker keeps the subscription and queues the QoS 1 publication. Result: message is delivered after reconnect.

Test 5 Persistent session, Subscribe QoS 1, Publish QoS 0: effective QoS is 0 for this publication, so it is not queued offline. Result: no offline message after reconnect.

# 9. Retained Message and Last Will

## 9.1 Retained Message

A retained publish stores the most recent retained payload for its topic at the broker. A later new subscription to that topic receives the retained message immediately. In the publisher, the --retained-demo option sets retain=True on iot/home_ID6233/retained_6233.

## 9.2 Last Will Message

The Last Will is registered before connecting. If the client loses the connection unexpectedly, the broker publishes the configured will payload on iot/home_ID6233/status_6233. The implementation uses QoS 1 and retain=True for the status message.

## 9.3 Keep Alive

Both clients use a 90-second keep-alive period as required. This controls the maximum interval used to maintain the MQTT connection and allows the broker to detect a lost client connection.

# 10. Validation evidence

Live validation was executed on the student's Windows PC against broker.hivemq.com:1883. The TCP connection succeeded and the automated MQTT runner completed all five tests. Tests 1, 2, 3 and 5 correctly showed no queued offline delivery, while Test 4 delivered the queued QoS 1 message after reconnect. Final result: 5/5 tests passed with 0 connection/runtime errors.

Figures 4–5 Live MQTT Tests 1–5 executed against broker.hivemq.com:1883. The run finished successfully with 5/5 tests passed and 0 connection/runtime errors Test 4 received the expected queued message after reconnect.

# 11. Running the submission files

Install Python 3 and the dependency:  py -m pip install -r requirements.txt

Run all five live tests:  py mqtt_test_runner_6233.py

For a manual test, open subscriber first, disconnect it after the subscription is registered, publish while it is offline, then reconnect the subscriber with --skip-subscribe.

For the retained-message demo, run the publisher with --retained-demo and then start a new subscriber with --retained-demo.

# 12. Submission package contents

Assignment_1_IoT_Itzhak_Davidov_6233.docx — report

mqtt_publisher_6233.py - publisher implementation

mqtt_subscriber_6233.py - subscriber implementation

mqtt_test_runner_6233.py - automated live Tests 1–5

offline_validation_6233.py - local configuration/protocol validation

requirements.txt — Python dependency

README.txt — run instructions

# 13. Conclusion

The prepared MQTT solution satisfies the Assignment 1 implementation requirements: unique 6233 client/topic identifiers, HiveMQ broker settings, Clean Session and QoS configurations for Tests 1-5, retained publishing, Last Will, and a 90-second keep-alive. The final live execution against broker.hivemq.com:1883 completed successfully with 5/5 tests passed and 0 connection/runtime errors, confirming the expected MQTT session behavior.

# Appendix – Full Source Code

Student: Itzhak Davidov | Last 4 digits: 6233

The complete source used for this assignment is included below and is also provided as separate files in the GitHub repository.

## mqtt_publisher_6233.py

001  """Assignment 1 - MQTT Publisher

002  Student: Itzhak Davidov | ID last 4 digits: 6233

003  """

004

005  import argparse

006  import threading

007  import time

008

009  import paho.mqtt.client as mqtt

010

011  BROKER = "broker.hivemq.com"

012  PORT = 1883

013  KEEPALIVE = 90

014  TOPIC = "iot/home_ID6233/sensor_6233"

015  RETAINED_TOPIC = "iot/home_ID6233/retained_6233"

016  WILL_TOPIC = "iot/home_ID6233/status_6233"

017

018  TESTS = {

019      1: {"clean_session": True,  "pub_qos": 0, "sub_qos": 0},

020      2: {"clean_session": False, "pub_qos": 0, "sub_qos": 0},

021      3: {"clean_session": True,  "pub_qos": 1, "sub_qos": 1},

022      4: {"clean_session": False, "pub_qos": 1, "sub_qos": 1},

023      5: {"clean_session": False, "pub_qos": 0, "sub_qos": 1},

024  }

025

026

027  def create_client(client_id: str, clean_session: bool):

028      """Create a Paho MQTT v3.1.1 client, compatible with Paho 1.x/2.x."""

029      try:

030          return mqtt.Client(

031              mqtt.CallbackAPIVersion.VERSION1,

032              client_id=client_id,

033              clean_session=clean_session,

034              protocol=mqtt.MQTTv311,

035          )

036      except AttributeError:

037          return mqtt.Client(

038              client_id=client_id,

039              clean_session=clean_session,

040              protocol=mqtt.MQTTv311,

041          )

042

043

044  def main():

045      parser = argparse.ArgumentParser(description="MQTT publisher for Assignment 1")

046      parser.add_argument("--test", type=int, choices=range(1, 6), default=4)

047      parser.add_argument("--message", default="Hello from Itzhak Davidov - 6233")

048      parser.add_argument("--retained-demo", action="store_true",

049                          help="Publish a retained message to the retained topic")

050      args = parser.parse_args()

051

052      cfg = TESTS[args.test]

053      connected = threading.Event()

054      client_id = f"IoT_ID_6233_PUB_T{args.test}"

055      client = create_client(client_id, cfg["clean_session"])

056

057      def on_connect(client, userdata, flags, rc):

058          print(f"CONNECTED rc={rc} broker={BROKER}:{PORT}")

059          connected.set()

060

061      def on_disconnect(client, userdata, rc):

062          print(f"DISCONNECTED rc={rc}")

063

064      client.on_connect = on_connect

065      client.on_disconnect = on_disconnect

066

067      # Last Will message: broker publishes it if this client disconnects unexpectedly.

068      client.will_set(

069          WILL_TOPIC,

070          payload="Publisher 6233 disconnected unexpectedly",

071          qos=1,

072          retain=True,

073      )

074

075      print(f"Student: Itzhak Davidov | suffix: 6233")

076      print(f"Test {args.test}: clean_session={cfg['clean_session']} pub_qos={cfg['pub_qos']}")

077      print(f"Client ID: {client_id}")

078      print(f"Keep Alive: {KEEPALIVE} seconds")

079      print(f"Connecting to {BROKER}:{PORT} ...")

080

081      client.connect(BROKER, PORT, keepalive=KEEPALIVE)

082      client.loop_start()

083      if not connected.wait(10):

084          client.loop_stop()

085          raise TimeoutError("Connection to broker timed out")

086

087      topic = RETAINED_TOPIC if args.retained_demo else TOPIC

088      retain = bool(args.retained_demo)

089      payload = "Retained message - Itzhak Davidov 6233" if retain else args.message

090

091      info = client.publish(topic, payload=payload, qos=cfg["pub_qos"], retain=retain)

092      info.wait_for_publish(timeout=10)

093      print(f"PUBLISHED topic={topic}")

094      print(f"payload={payload!r} qos={cfg['pub_qos']} retain={retain}")

095

096      time.sleep(1)

097      client.disconnect()

098      client.loop_stop()

099      print("Publisher finished successfully")

100

101

102  if __name__ == "__main__":

103      main()

## mqtt_subscriber_6233.py

001  """Assignment 1 - MQTT Subscriber

002  Student: Itzhak Davidov | ID last 4 digits: 6233

003  """

004

005  import argparse

006  import threading

007  import time

008

009  import paho.mqtt.client as mqtt

010

011  BROKER = "broker.hivemq.com"

012  PORT = 1883

013  KEEPALIVE = 90

014  TOPIC = "iot/home_ID6233/sensor_6233"

015  RETAINED_TOPIC = "iot/home_ID6233/retained_6233"

016  WILL_TOPIC = "iot/home_ID6233/status_6233"

017

018  TESTS = {

019      1: {"clean_session": True,  "pub_qos": 0, "sub_qos": 0},

020      2: {"clean_session": False, "pub_qos": 0, "sub_qos": 0},

021      3: {"clean_session": True,  "pub_qos": 1, "sub_qos": 1},

022      4: {"clean_session": False, "pub_qos": 1, "sub_qos": 1},

023      5: {"clean_session": False, "pub_qos": 0, "sub_qos": 1},

024  }

025

026

027  def create_client(client_id: str, clean_session: bool):

028      """Create a Paho MQTT v3.1.1 client, compatible with Paho 1.x/2.x."""

029      try:

030          return mqtt.Client(

031              mqtt.CallbackAPIVersion.VERSION1,

032              client_id=client_id,

033              clean_session=clean_session,

034              protocol=mqtt.MQTTv311,

035          )

036      except AttributeError:

037          return mqtt.Client(

038              client_id=client_id,

039              clean_session=clean_session,

040              protocol=mqtt.MQTTv311,

041          )

042

043

044  def main():

045      parser = argparse.ArgumentParser(description="MQTT subscriber for Assignment 1")

046      parser.add_argument("--test", type=int, choices=range(1, 6), default=4)

047      parser.add_argument("--duration", type=int, default=15)

048      parser.add_argument("--retained-demo", action="store_true")

049      parser.add_argument("--skip-subscribe", action="store_true",

050                          help="Reconnect without SUBSCRIBE to verify a persistent session")

051      args = parser.parse_args()

052

053      cfg = TESTS[args.test]

054      connected = threading.Event()

055      received = []

056      client_id = f"IoT_ID_6233_SUB_T{args.test}"

057      client = create_client(client_id, cfg["clean_session"])

058

059      def on_connect(client, userdata, flags, rc):

060          print(f"CONNECTED rc={rc} broker={BROKER}:{PORT}")

061          connected.set()

062          if not args.skip_subscribe:

063              topic = RETAINED_TOPIC if args.retained_demo else TOPIC

064              client.subscribe(topic, qos=cfg["sub_qos"])

065              print(f"SUBSCRIBED topic={topic} qos={cfg['sub_qos']}")

066          else:

067              print("SUBSCRIBE skipped - checking stored persistent session")

068

069      def on_message(client, userdata, msg):

070          payload = msg.payload.decode("utf-8", errors="replace")

071          received.append((msg.topic, payload, msg.qos, msg.retain))

072          print(f"MESSAGE topic={msg.topic} payload={payload!r} qos={msg.qos} retain={msg.retain}")

073

074      def on_disconnect(client, userdata, rc):

075          print(f"DISCONNECTED rc={rc}")

076

077      client.on_connect = on_connect

078      client.on_message = on_message

079      client.on_disconnect = on_disconnect

080      client.will_set(

081          WILL_TOPIC,

082          payload="Subscriber 6233 disconnected unexpectedly",

083          qos=1,

084          retain=True,

085      )

086

087      print(f"Student: Itzhak Davidov | suffix: 6233")

088      print(f"Test {args.test}: clean_session={cfg['clean_session']} sub_qos={cfg['sub_qos']}")

089      print(f"Client ID: {client_id}")

090      print(f"Keep Alive: {KEEPALIVE} seconds")

091      print(f"Connecting to {BROKER}:{PORT} ...")

092

093      client.connect(BROKER, PORT, keepalive=KEEPALIVE)

094      client.loop_start()

095      if not connected.wait(10):

096          client.loop_stop()

097          raise TimeoutError("Connection to broker timed out")

098

099      time.sleep(args.duration)

100      print(f"Messages received: {len(received)}")

101      client.disconnect()

102      client.loop_stop()

103      print("Subscriber finished successfully")

104

105

106  if __name__ == "__main__":

107      main()

## mqtt_test_runner_6233.py

001  """Reliable MQTT Tests 1-5 for Assignment 1.

002  Student: Itzhak Davidov | ID suffix: 6233

003  Broker: broker.hivemq.com:1883 | MQTT 3.1.1 | Keep Alive: 90 s

004  """

005

006  import socket

007  import threading

008  import time

009  import uuid

010  import warnings

011  from pathlib import Path

012

013  import paho.mqtt.client as mqtt

014

015  warnings.filterwarnings("ignore", category=DeprecationWarning)

016

017  BROKER = "broker.hivemq.com"

018  PORT = 1883

019  KEEPALIVE = 90

020  TOPIC_PREFIX = "iot/home_ID6233/tests"

021  RESULTS_FILE = Path(__file__).with_name("live_test_results_6233.txt")

022

023  # test_no: (clean_session, subscribe_qos, publish_qos, expect_offline_delivery)

024  TESTS = {

025      1: (True,  0, 0, False),

026      2: (False, 0, 0, False),

027      3: (True,  1, 1, False),

028      4: (False, 1, 1, True),

029      5: (False, 1, 0, False),

030  }

031

032  _LOG_LINES = []

033

034

035  def log(text=""):

036      print(text, flush=True)

037      _LOG_LINES.append(str(text))

038      RESULTS_FILE.write_text("\n".join(_LOG_LINES) + "\n", encoding="utf-8")

039

040

041  def make_client(client_id, clean_session):

042      """Create an MQTT 3.1.1 client using the compatibility callback API."""

043      try:

044          return mqtt.Client(

045              mqtt.CallbackAPIVersion.VERSION1,

046              client_id=client_id,

047              clean_session=clean_session,

048              protocol=mqtt.MQTTv311,

049          )

050      except (AttributeError, TypeError):

051          return mqtt.Client(

052              client_id=client_id,

053              clean_session=clean_session,

054              protocol=mqtt.MQTTv311,

055          )

056

057

058  def resolve_ipv4():

059      """Resolve the public broker to IPv4 for a predictable Windows connection path."""

060      return socket.gethostbyname(BROKER)

061

062

063  def connect_wait(client, host, timeout=20, attempts=3):

064      """Connect and wait for MQTT CONNACK, retrying transient public-broker failures."""

065      last_error = None

066      for attempt in range(1, attempts + 1):

067          connected = threading.Event()

068          result = {"rc": None}

069

070          def on_connect(c, u, flags, rc):

071              result["rc"] = rc

072              connected.set()

073

074          client.on_connect = on_connect

075          loop_started = False

076          try:

077              rc = client.connect(host, PORT, keepalive=KEEPALIVE)

078              if rc != mqtt.MQTT_ERR_SUCCESS:

079                  raise RuntimeError(f"connect() returned {rc}")

080              client.loop_start()

081              loop_started = True

082              if not connected.wait(timeout):

083                  raise TimeoutError(f"no MQTT CONNACK within {timeout}s")

084              if result["rc"] != 0:

085                  raise ConnectionError(f"broker rejected CONNECT, rc={result['rc']}")

086              return

087          except Exception as exc:

088              last_error = exc

089              try:

090                  client.disconnect()

091              except Exception:

092                  pass

093              if loop_started:

094                  try:

095                      client.loop_stop()

096                  except Exception:

097                      pass

098              if attempt < attempts:

099                  time.sleep(2 * attempt)

100

101      raise TimeoutError(f"MQTT connection failed after {attempts} attempts: {last_error}")

102

103

104  def subscribe_wait(client, topic, qos, timeout=8):

105      """Subscribe and wait for SUBACK so the broker has definitely stored the subscription."""

106      ack = threading.Event()

107

108      def on_subscribe(c, u, mid, granted_qos):

109          ack.set()

110

111      client.on_subscribe = on_subscribe

112      rc, _mid = client.subscribe(topic, qos=qos)

113      if rc != mqtt.MQTT_ERR_SUCCESS:

114          raise RuntimeError(f"subscribe() returned {rc}")

115      if not ack.wait(timeout):

116          raise TimeoutError("SUBACK timeout")

117

118

119  def disconnect_cleanly(client, wait=0.8):

120      """Give the network loop time to transmit DISCONNECT before stopping it."""

121      try:

122          client.disconnect()

123          time.sleep(wait)

124      finally:

125          try:

126              client.loop_stop()

127          except Exception:

128              pass

129

130

131  def run_test(test_no, broker_ip, run_token):

132      clean, sub_qos, pub_qos, expected = TESTS[test_no]

133

134      # Unique per program run, but every ID/topic still ends in 6233 as requested.

135      sub_id = f"IoT_ItzhakD_T{test_no}_{run_token}_6233"

136      pub_id = f"IoT_ItzhakD_PUB{test_no}_{run_token}_6233"

137      topic = f"{TOPIC_PREFIX}/test{test_no}_{run_token}_6233"

138      payload = f"offline-message-test-{test_no}-6233"

139

140      log(f"Test {test_no}: clean_session={clean}, SUB QoS={sub_qos}, PUB QoS={pub_qos}")

141

142      # Phase A: establish subscription and then go offline.

143      sub = make_client(sub_id, clean)

144      connect_wait(sub, broker_ip)

145      subscribe_wait(sub, topic, sub_qos)

146      log("  A) subscriber connected + SUBACK received")

147      disconnect_cleanly(sub)

148      time.sleep(1.2)

149

150      # Phase B: publish while subscriber is offline.

151      pub = make_client(pub_id, True)

152      connect_wait(pub, broker_ip)

153      info = pub.publish(topic, payload=payload, qos=pub_qos, retain=False)

154      if pub_qos > 0:

155          info.wait_for_publish(timeout=10)

156      else:

157          time.sleep(0.8)

158      log("  B) publisher sent the offline message")

159      disconnect_cleanly(pub)

160      time.sleep(1.5)

161

162      # Phase C: reconnect SAME subscriber ID. Do not subscribe again.

163      got = []

164      received = threading.Event()

165      sub2 = make_client(sub_id, clean)

166

167      def on_message(c, u, m):

168          got.append(m.payload.decode(errors="replace"))

169          received.set()

170

171      sub2.on_message = on_message

172      connect_wait(sub2, broker_ip)

173      received.wait(5.0)

174      disconnect_cleanly(sub2)

175

176      actual = bool(got)

177      ok = actual == expected

178      log(f"  C) reconnect without re-subscribing -> message_received={actual}")

179      log(f"  Expected offline delivery={expected} -> {'PASS' if ok else 'FAIL'}")

180      if got:

181          log(f"  Received: {got}")

182      log()

183      return ok

184

185

186  def main():

187      run_token = uuid.uuid4().hex[:6]

188      _LOG_LINES.clear()

189      if RESULTS_FILE.exists():

190          RESULTS_FILE.unlink()

191

192      log("Assignment 1 MQTT automated test runner - FIXED")

193      log("Student: Itzhak Davidov | suffix: 6233")

194      log(f"Broker: {BROKER}:{PORT} | Keep Alive: {KEEPALIVE}s")

195      log(f"Run token: {run_token}")

196      log("-" * 72)

197

198      broker_ip = resolve_ipv4()

199      log(f"Resolved broker IPv4: {broker_ip}")

200      try:

201          with socket.create_connection((broker_ip, PORT), timeout=7):

202              log("TCP connectivity check: PASS")

203      except Exception as exc:

204          log(f"TCP connectivity check: FAIL ({exc})")

205          log("The MQTT tests were not started.")

206          return 2

207

208      log("-" * 72)

209      passed = 0

210      errors = 0

211      for n in range(1, 6):

212          try:

213              ok = run_test(n, broker_ip, run_token)

214              passed += int(ok)

215          except Exception as exc:

216              errors += 1

217              log(f"Test {n}: ERROR -> {type(exc).__name__}: {exc}")

218              log("Continuing to the next test...")

219              log()

220              time.sleep(2)

221

222      log("-" * 72)

223      log(f"FINAL RESULT: {passed}/5 tests passed | connection/runtime errors: {errors}")

224      log(f"Results saved to: {RESULTS_FILE.name}")

225      if passed == 5 and errors == 0:

226          log("STATUS: READY FOR SCREENSHOT")

227          return 0

228      log("STATUS: Send this screen/result file to ChatGPT for the next fix.")

229      return 1

230

231

232  if __name__ == "__main__":

233      raise SystemExit(main())

## offline_validation_6233.py

001  """Offline validation for Assignment 1 files.

002  This validates required configuration and the expected MQTT v3.1.1 session outcomes.

003  It does not contact the public broker.

004  """

005  from pathlib import Path

006

007  ROOT = Path(__file__).resolve().parent

008  files = [ROOT / "mqtt_publisher_6233.py", ROOT / "mqtt_subscriber_6233.py"]

009  required = {

010      'broker.hivemq.com': 'HiveMQ broker',

011      'PORT = 1883': 'TCP port 1883',

012      'KEEPALIVE = 90': 'keep-alive 90 seconds',

013      '6233': 'student ID suffix',

014      'will_set': 'Last Will message',

015      'retain=True': 'retained flag',

016  }

017

018  print("Assignment 1 - Offline validation")

019  print("Student: Itzhak Davidov | suffix: 6233")

020  print("=" * 68)

021  for f in files:

022      text = f.read_text(encoding='utf-8')

023      print(f"\n{f.name}")

024      for needle, label in required.items():

025          ok = needle in text

026          print(f"  [{'PASS' if ok else 'FAIL'}] {label}")

027

028  print("\nMQTT offline-delivery test matrix")

029  print("Test  CleanSession  SubQoS  PubQoS  Expected after reconnect")

030  rows = [

031      (1, True,  0, 0, 'NO'),

032      (2, False, 0, 0, 'NO'),

033      (3, True,  1, 1, 'NO'),

034      (4, False, 1, 1, 'YES'),

035      (5, False, 1, 0, 'NO'),

036  ]

037  for n, clean, sq, pq, expected in rows:

038      print(f" {n:<4} {str(clean):<13} {sq:<7} {pq:<7} {expected}")

039

040  print("\n[PASS] Configuration is ready for broker execution.")

041  print("[INFO] Run mqtt_test_runner_6233.py on an Internet-connected computer")

042  print("       to capture live HiveMQ results.")