# Assignment #2 - IoT
Hands-On 4

Itzhak Davidov

Last 4 digits: 6233

MQTT continuous subscriber, DHT data collection, RELAY control and BUTTON event handling

# 1. Assignment goals and setup

The assignment was implemented according to Hands-On 4. The client connects to the public HiveMQ broker, runs a continuous MQTT subscriber, monitors the assigned cube group, and implements event handlers for DHT, RELAY and BUTTON. Tests 1–3 were executed live against broker.hivemq.com on port 1883.

## MQTT configuration

| Item | Value | Purpose |
| --- | --- | --- |
| Broker | broker.hivemq.com:1883 | Public MQTT broker |
| DHT status | iot/6233/cubes/dht/sts | Temperature and humidity samples |
| RELAY command | iot/6233/cubes/relay/cmd | ON/OFF commands |
| RELAY status | iot/6233/cubes/relay/sts | Relay state confirmation |
| BUTTON status | iot/6233/cubes/button/sts | Button press events |

# 2. Test 1 - DHT data collection

Requirement: monitor the DHT emulator for 10 minutes and collect 20 temperature and humidity samples. The DHT emulator publishes a new sample every 30 seconds and the Assignment 2 client stores every received sample in dht_samples_6233.csv.

PASS – 20/20 DHT samples were received and saved successfully.

Observed live data: temperature range 22.1–23.0 °C (average 22.54 °C) and humidity range 74.4–76.4% (average 75.27%).

# 3. Test 2 - RELAY control

Requirement: send an ON command to the RELAY, observe the state change, then send an OFF command and observe the second state change. The live execution below shows both commands and both status confirmations.

PASS – RELAY changed to ON, confirmed ON, then changed to OFF and confirmed OFF.

# 4. Test 3 – BUTTON event detector with RELAY

Requirement: detect each BUTTON push immediately and use the RELAY as a paired device, toggling the RELAY state every time the BUTTON is pushed. During the live run, three BUTTON events were detected and the client issued the sequence ON → OFF → ON.

PASS – every BUTTON event was detected and toggled the RELAY exactly once.

# 5. Main MQTT implementation

The main client subscribes continuously to DHT, BUTTON and RELAY status topics. DHT messages are parsed and stored, RELAY status messages update the known relay state, and BUTTON events call the relay toggle logic.

def on_message(self, client, userdata, msg):

raw = msg.payload.decode("utf-8", "ignore")

if msg.topic == DHT_TOPIC and self.collect_dht:

self.handle_dht(raw)

elif msg.topic == BUTTON_TOPIC:

self.handle_button(raw)

elif msg.topic == RELAY_STATUS_TOPIC:

self.handle_relay_status(raw)

def handle_button(self, raw):

data = json.loads(raw)

if int(data.get("value", 0)) == 1:

self.button_events += 1

if self.react_to_button:

self.set_relay(not self.relay_on)

def set_relay(self, on):

self.relay_on = bool(on)

payload = json.dumps({"value": 1 if on else 0})

self.client.publish(RELAY_CMD_TOPIC, payload, qos=0)

# 6. Test summary

| Test | Goal | Observed result | Status |
| --- | --- | --- | --- |
| 1 | Collect 20 DHT samples in 10 min | 20/20 samples received; CSV saved | PASS |
| 2 | Control RELAY ON then OFF | Both state changes and confirmations observed | PASS |
| 3 | Detect BUTTON and toggle RELAY | Three events produced ON → OFF → ON | PASS |

Tests 4–5 were not included because the assignment slides explicitly mark them as “only in class”.

# 7. Conclusion

Assignment 2 was completed for student identifier 6233. The MQTT client connected successfully to HiveMQ, collected the required 20 DHT samples, controlled the RELAY in both directions, and detected BUTTON events that toggled the RELAY state. The screenshots in this report are from the live execution of Tests 1–3.

# Appendix – Full Source Code

Student: Itzhak Davidov | Last 4 digits: 6233

The complete source used for this assignment is included below and is also provided as separate files in the GitHub repository.

## assignment2_client_6233.py

001  # Assignment 2 - IoT / MQTT

002  # Student: Itzhak Davidov | last 4 digits: 6233

003

004  import argparse

005  import csv

006  import json

007  import time

008  from datetime import datetime

009  import paho.mqtt.client as mqtt

010

011  from mqtt_init_6233 import (

012      BROKER, PORT, KEEPALIVE,

013      DHT_TOPIC, BUTTON_TOPIC, RELAY_CMD_TOPIC, RELAY_STATUS_TOPIC

014  )

015

016  def make_client(client_id):

017      try:

018          return mqtt.Client(mqtt.CallbackAPIVersion.VERSION1, client_id=client_id, clean_session=True)

019      except Exception:

020          return mqtt.Client(client_id=client_id, clean_session=True)

021

022  class Assignment2Client:

023      def __init__(self):

024          self.client = make_client("IOT_ASSIGNMENT2_6233")

025          self.client.on_connect = self.on_connect

026          self.client.on_message = self.on_message

027          self.samples = []

028          self.relay_on = False

029          self.button_events = 0

030          self.collect_dht = False

031          self.react_to_button = False

032

033      def connect(self):

034          print(f"Connecting to {BROKER}:{PORT} ...")

035          self.client.connect(BROKER, PORT, KEEPALIVE)

036          self.client.loop_start()

037

038      def close(self):

039          self.client.loop_stop()

040          self.client.disconnect()

041

042      def on_connect(self, client, userdata, flags, rc):

043          if rc == 0:

044              print("connected OK")

045              client.subscribe([

046                  (DHT_TOPIC, 0),

047                  (BUTTON_TOPIC, 0),

048                  (RELAY_STATUS_TOPIC, 0),

049              ])

050              print("Subscribed to cube group 6233")

051          else:

052              print("Bad connection. rc =", rc)

053

054      def on_message(self, client, userdata, msg):

055          raw = msg.payload.decode("utf-8", "ignore")

056          print(f"RX {msg.topic}: {raw}")

057

058          if msg.topic == DHT_TOPIC and self.collect_dht:

059              self.handle_dht(raw)

060          elif msg.topic == BUTTON_TOPIC:

061              self.handle_button(raw)

062          elif msg.topic == RELAY_STATUS_TOPIC:

063              self.handle_relay_status(raw)

064

065      def handle_dht(self, raw):

066          try:

067              data = json.loads(raw)

068              t = float(data["temperature"])

069              h = float(data["humidity"])

070          except Exception:

071              # Backwards-compatible parser for old emulator:

072              # "Temperature: 22.5 Humidity: 75.1"

073              parts = raw.replace(":", "").split()

074              t = float(parts[1])

075              h = float(parts[3])

076

077          row = (datetime.now().isoformat(timespec="seconds"), t, h)

078          self.samples.append(row)

079          print(f"DHT sample {len(self.samples):02d}/20 -> T={t:.1f} C, H={h:.1f}%")

080

081      def handle_button(self, raw):

082          pressed = False

083          try:

084              data = json.loads(raw)

085              pressed = int(data.get("value", 0)) == 1

086          except Exception:

087              pressed = "1" in raw

088

089          if pressed:

090              self.button_events += 1

091              print(f"BUTTON event #{self.button_events}")

092              if self.react_to_button:

093                  self.set_relay(not self.relay_on)

094

095      def handle_relay_status(self, raw):

096          try:

097              data = json.loads(raw)

098              self.relay_on = str(data.get("relay", "")).upper() == "ON"

099          except Exception:

100              self.relay_on = raw.strip().upper() in {"1", "ON", "TRUE"}

101          print("RELAY confirmed:", "ON" if self.relay_on else "OFF")

102

103      def set_relay(self, on):

104          self.relay_on = bool(on)

105          payload = json.dumps({"value": 1 if on else 0})

106          self.client.publish(RELAY_CMD_TOPIC, payload, qos=0)

107          print("TX relay command:", "ON" if on else "OFF")

108

109      def test1(self):

110          print("\n=== TEST 1: DHT - 20 samples / 10 minutes ===")

111          self.collect_dht = True

112          deadline = time.time() + 610

113          while len(self.samples) < 20 and time.time() < deadline:

114              time.sleep(1)

115          self.collect_dht = False

116

117          with open("dht_samples_6233.csv", "w", newline="", encoding="utf-8") as f:

118              w = csv.writer(f)

119              w.writerow(["timestamp", "temperature_c", "humidity_pct"])

120              w.writerows(self.samples)

121          print(f"Saved {len(self.samples)} samples to dht_samples_6233.csv")

122

123      def test2(self):

124          print("\n=== TEST 2: RELAY ON / OFF ===")

125          self.set_relay(True)

126          time.sleep(3)

127          self.set_relay(False)

128          time.sleep(3)

129          print("TEST 2 complete")

130

131      def test3(self):

132          print("\n=== TEST 3: BUTTON -> RELAY toggle ===")

133          self.react_to_button = True

134          print("Press the BUTTON emulator. Each press toggles the RELAY.")

135          time.sleep(120)

136          self.react_to_button = False

137          print(f"TEST 3 complete. Button events detected: {self.button_events}")

138

139  def main():

140      parser = argparse.ArgumentParser()

141      parser.add_argument("--test", choices=["1", "2", "3"], required=True)

142      args = parser.parse_args()

143

144      app = Assignment2Client()

145      app.connect()

146      time.sleep(2)

147      try:

148          {"1": app.test1, "2": app.test2, "3": app.test3}[args.test]()

149      finally:

150          app.close()

151

152  if __name__ == "__main__":

153      main()

## DHT_6233.py

001  # Assignment 2 - IoT / MQTT

002  # Student: Itzhak Davidov | last 4 digits: 6233

003

004  import json

005  import random

006  import sys

007  from PyQt5 import QtCore, QtWidgets

008  import paho.mqtt.client as mqtt

009

010  BROKER = "broker.hivemq.com"

011  PORT = 1883

012  KEEPALIVE = 90

013  DHT_TOPIC = "iot/6233/cubes/dht/sts"

014

015

016  def make_client(client_id):

017      # Use callback API v1 to stay compatible with the assignment callbacks.

018      return mqtt.Client(

019          mqtt.CallbackAPIVersion.VERSION1,

020          client_id=client_id,

021          clean_session=True,

022      )

023

024

025  class DHTWindow(QtWidgets.QWidget):

026      mqtt_connected = QtCore.pyqtSignal(int)

027      mqtt_disconnected = QtCore.pyqtSignal(int)

028

029      def __init__(self):

030          super().__init__()

031          self.setWindowTitle("DHT Emulator - 6233")

032

033          self.client = make_client("IOT_DHT_6233")

034          self.client.on_connect = self.on_connect

035          self.client.on_disconnect = self.on_disconnect

036

037          self.status = QtWidgets.QLabel("Disconnected")

038          self.temp = QtWidgets.QLabel("--")

039          self.hum = QtWidgets.QLabel("--")

040          self.btn = QtWidgets.QPushButton("Connect")

041          self.btn.clicked.connect(self.connect_mqtt)

042

043          form = QtWidgets.QFormLayout(self)

044          form.addRow("Broker", QtWidgets.QLabel(f"{BROKER}:{PORT}"))

045          form.addRow("Topic", QtWidgets.QLabel(DHT_TOPIC))

046          form.addRow("Status", self.status)

047          form.addRow("Temperature °C", self.temp)

048          form.addRow("Humidity %", self.hum)

049          form.addRow(self.btn)

050

051          # QTimer must be started from the Qt GUI thread.

052          self.timer = QtCore.QTimer(self)

053          self.timer.setInterval(30_000)  # 20 samples in 10 minutes

054          self.timer.timeout.connect(self.publish_sample)

055

056          # MQTT callbacks run on Paho's network thread, so marshal GUI work

057          # back to the Qt thread through signals.

058          self.mqtt_connected.connect(self.handle_connected)

059          self.mqtt_disconnected.connect(self.handle_disconnected)

060

061      def connect_mqtt(self):

062          self.status.setText("Connecting...")

063          self.btn.setEnabled(False)

064          try:

065              self.client.connect(BROKER, PORT, KEEPALIVE)

066              self.client.loop_start()

067          except Exception as exc:

068              self.status.setText(f"Connection failed: {exc}")

069              self.btn.setEnabled(True)

070

071      # Paho thread callback: do not touch Qt widgets/timers directly here.

072      def on_connect(self, client, userdata, flags, rc):

073          self.mqtt_connected.emit(int(rc))

074

075      # Paho thread callback: do not touch Qt widgets/timers directly here.

076      def on_disconnect(self, client, userdata, rc):

077          self.mqtt_disconnected.emit(int(rc))

078

079      @QtCore.pyqtSlot(int)

080      def handle_connected(self, rc):

081          if rc == 0:

082              self.status.setText("Connected")

083              self.btn.setEnabled(False)

084              self.publish_sample()   # first sample immediately

085              self.timer.start()     # then every 30 seconds

086          else:

087              self.status.setText(f"Connection error: {rc}")

088              self.btn.setEnabled(True)

089

090      @QtCore.pyqtSlot(int)

091      def handle_disconnected(self, rc):

092          self.timer.stop()

093          self.status.setText("Disconnected")

094          self.btn.setEnabled(True)

095

096      @QtCore.pyqtSlot()

097      def publish_sample(self):

098          temperature = round(22.0 + random.uniform(0.1, 1.0), 1)

099          humidity = round(74.0 + random.uniform(0.1, 2.4), 1)

100          payload = json.dumps({"temperature": temperature, "humidity": humidity})

101          self.client.publish(DHT_TOPIC, payload, qos=0)

102          self.temp.setText(str(temperature))

103          self.hum.setText(str(humidity))

104          print(f"DHT -> {DHT_TOPIC}: {payload}", flush=True)

105

106      def closeEvent(self, event):

107          self.timer.stop()

108          try:

109              self.client.loop_stop()

110              self.client.disconnect()

111          except Exception:

112              pass

113          event.accept()

114

115

116  if __name__ == "__main__":

117      app = QtWidgets.QApplication(sys.argv)

118      w = DHTWindow()

119      w.resize(520, 220)

120      w.show()

121      sys.exit(app.exec_())

## RELAY_6233.py

001  # Assignment 2 - IoT / MQTT

002  # Student: Itzhak Davidov | last 4 digits: 6233

003

004  import json

005  import sys

006  from PyQt5 import QtWidgets

007  import paho.mqtt.client as mqtt

008  from mqtt_init_6233 import BROKER, PORT, KEEPALIVE, RELAY_CMD_TOPIC, RELAY_STATUS_TOPIC

009

010  def make_client(client_id):

011      try:

012          return mqtt.Client(mqtt.CallbackAPIVersion.VERSION1, client_id=client_id, clean_session=True)

013      except Exception:

014          return mqtt.Client(client_id=client_id, clean_session=True)

015

016  class RelayWindow(QtWidgets.QWidget):

017      def __init__(self):

018          super().__init__()

019          self.setWindowTitle("RELAY Emulator - 6233")

020          self.state = False

021

022          self.client = make_client("IOT_RELAY_6233")

023          self.client.on_connect = self.on_connect

024          self.client.on_message = self.on_message

025

026          self.status = QtWidgets.QLabel("Disconnected")

027          self.state_label = QtWidgets.QLabel("OFF")

028          self.btn = QtWidgets.QPushButton("Connect")

029          self.btn.clicked.connect(self.connect_mqtt)

030

031          form = QtWidgets.QFormLayout(self)

032          form.addRow("Broker", QtWidgets.QLabel(f"{BROKER}:{PORT}"))

033          form.addRow("Command topic", QtWidgets.QLabel(RELAY_CMD_TOPIC))

034          form.addRow("Status topic", QtWidgets.QLabel(RELAY_STATUS_TOPIC))

035          form.addRow("MQTT", self.status)

036          form.addRow("Relay", self.state_label)

037          form.addRow(self.btn)

038          self.update_style()

039

040      def connect_mqtt(self):

041          self.status.setText("Connecting...")

042          self.client.connect(BROKER, PORT, KEEPALIVE)

043          self.client.loop_start()

044

045      def on_connect(self, client, userdata, flags, rc):

046          if rc == 0:

047              self.status.setText("Connected")

048              self.btn.setEnabled(False)

049              self.client.subscribe(RELAY_CMD_TOPIC, qos=0)

050              self.publish_status()

051          else:

052              self.status.setText(f"Connection error: {rc}")

053

054      def on_message(self, client, userdata, msg):

055          raw = msg.payload.decode("utf-8", "ignore").strip()

056          value = raw.upper()

057          try:

058              data = json.loads(raw)

059              value = str(data.get("value", data.get("relay", raw))).upper()

060          except Exception:

061              pass

062

063          if value in {"1", "ON", "TRUE"}:

064              self.set_state(True)

065          elif value in {"0", "OFF", "FALSE"}:

066              self.set_state(False)

067          elif value == "TOGGLE":

068              self.set_state(not self.state)

069          else:

070              print("Unknown relay command:", raw)

071

072      def set_state(self, on):

073          self.state = bool(on)

074          self.state_label.setText("ON" if self.state else "OFF")

075          self.update_style()

076          self.publish_status()

077          print("RELAY state ->", "ON" if self.state else "OFF")

078

079      def update_style(self):

080          self.state_label.setStyleSheet(

081              "font-size: 26px; font-weight: bold; "

082              + ("background:#b7f7bd;" if self.state else "background:#e5e5e5;")

083          )

084

085      def publish_status(self):

086          if self.client.is_connected():

087              self.client.publish(

088                  RELAY_STATUS_TOPIC,

089                  json.dumps({"relay": "ON" if self.state else "OFF"}),

090                  qos=0,

091                  retain=True,

092              )

093

094  if __name__ == "__main__":

095      app = QtWidgets.QApplication(sys.argv)

096      w = RelayWindow()

097      w.resize(600, 230)

098      w.show()

099      sys.exit(app.exec_())

## BUTTON_6233.py

001  # Assignment 2 - IoT / MQTT

002  # Student: Itzhak Davidov | last 4 digits: 6233

003

004  import json

005  import sys

006  from PyQt5 import QtWidgets

007  import paho.mqtt.client as mqtt

008  from mqtt_init_6233 import BROKER, PORT, KEEPALIVE, BUTTON_TOPIC

009

010  def make_client(client_id):

011      try:

012          return mqtt.Client(mqtt.CallbackAPIVersion.VERSION1, client_id=client_id, clean_session=True)

013      except Exception:

014          return mqtt.Client(client_id=client_id, clean_session=True)

015

016  class ButtonWindow(QtWidgets.QWidget):

017      def __init__(self):

018          super().__init__()

019          self.setWindowTitle("BUTTON Emulator - 6233")

020          self.client = make_client("IOT_BUTTON_6233")

021          self.client.on_connect = self.on_connect

022

023          self.status = QtWidgets.QLabel("Disconnected")

024          self.connect_btn = QtWidgets.QPushButton("Connect")

025          self.connect_btn.clicked.connect(self.connect_mqtt)

026          self.push_btn = QtWidgets.QPushButton("PUSH BUTTON")

027          self.push_btn.setEnabled(False)

028          self.push_btn.clicked.connect(self.push_button)

029

030          form = QtWidgets.QFormLayout(self)

031          form.addRow("Broker", QtWidgets.QLabel(f"{BROKER}:{PORT}"))

032          form.addRow("Topic", QtWidgets.QLabel(BUTTON_TOPIC))

033          form.addRow("MQTT", self.status)

034          form.addRow(self.connect_btn)

035          form.addRow(self.push_btn)

036

037      def connect_mqtt(self):

038          self.status.setText("Connecting...")

039          self.client.connect(BROKER, PORT, KEEPALIVE)

040          self.client.loop_start()

041

042      def on_connect(self, client, userdata, flags, rc):

043          if rc == 0:

044              self.status.setText("Connected")

045              self.connect_btn.setEnabled(False)

046              self.push_btn.setEnabled(True)

047          else:

048              self.status.setText(f"Connection error: {rc}")

049

050      def push_button(self):

051          payload = json.dumps({"value": 1})

052          self.client.publish(BUTTON_TOPIC, payload, qos=0)

053          print(f"BUTTON -> {BUTTON_TOPIC}: {payload}")

054

055  if __name__ == "__main__":

056      app = QtWidgets.QApplication(sys.argv)

057      w = ButtonWindow()

058      w.resize(520, 210)

059      w.show()

060      sys.exit(app.exec_())

## mqtt_init_6233.py

001  # Assignment 2 - IoT / MQTT configuration

002  # Student: Itzhak Davidov | last 4 digits: 6233

003

004  BROKER = "broker.hivemq.com"

005  PORT = 1883

006  KEEPALIVE = 60

007

008  BASE_TOPIC = "iot/6233/cubes"

009  DHT_TOPIC = f"{BASE_TOPIC}/dht/sts"

010  BUTTON_TOPIC = f"{BASE_TOPIC}/button/sts"

011  RELAY_CMD_TOPIC = f"{BASE_TOPIC}/relay/cmd"

012  RELAY_STATUS_TOPIC = f"{BASE_TOPIC}/relay/sts"