Assignment 3

MQTT GUI + Event Handler (Hands-On 6)

| Student | Itzhak Davidov |
| --- | --- |
| Last 4 digits | 6233 |
| Technology | Python, PyQt5, MQTT (Paho) |
| Live verification broker | broker.hivemq.com : 1883 |

Submission Report

The application implements an editable MQTT client GUI, continuous subscriber, publisher/device control, and an automatic sensor event handler with a Relay reaction.

# 1. Assignment Requirements and Implementation

Hands-On 6 requires a PyQt5 GUI that can create an MQTT client session, subscribe continuously, control devices, and react automatically to sensor data. The implemented application covers the requested fields and behavior.

| Requirement | Implementation |
| --- | --- |
| Broker IP / host and port | Editable fields + two presets: HiveMQ Demo and the course White Cube preset. |
| Running time | Editable seconds value; 0 means unlimited. A positive value schedules automatic disconnect. |
| Topic change | Editable subscribe topic and publish topic. |
| Client name | Editable unique Client ID with 6233 embedded. |
| Username / password | Editable authentication fields. |
| Connection status | Visible CONNECTED / DISCONNECTED indicator. |
| Continuous subscriber | Subscribes to iot/6233/cubes/# and displays incoming MQTT messages in the application log. |
| Event handler | DHT temperature events are parsed and compared with a configurable threshold. |
| Reaction | Relay command is sent automatically: temperature >= threshold -> ON, temperature < threshold -> OFF. |
| Device control | Manual publish and Relay ON/OFF controls are also included. |

# 2. MQTT Topics Used

Base topic: iot/6233/cubes

DHT status: iot/6233/cubes/dht/sts

Button status: iot/6233/cubes/button/sts

Relay command: iot/6233/cubes/relay/cmd

Relay status: iot/6233/cubes/relay/sts

Continuous subscription: iot/6233/cubes/#

# 3. GUI and Live MQTT Connection

The application was executed on Windows and connected successfully to the public HiveMQ broker on port 1883 for live verification. The GUI contains all configurable session fields requested by the assignment. The course White Cube server is also available as a preset in the code and GUI.

# 4. Event Handler Logic

Incoming MQTT messages are routed by topic. For DHT data, the JSON payload is parsed, the temperature is compared with the GUI threshold, and the application automatically publishes a Relay command. Repeated commands are suppressed while readings remain on the same side of the threshold.

if mode.startswith("DHT") and topic == DHT_TOPIC:
    self.handle_dht_event(payload)

threshold = float(self.threshold.value())
desired = temp >= threshold
if self.last_event_command != desired:
    self.set_relay(desired, "DHT event handler")
    self.last_event_command = desired

# 5. Live Test - High Temperature Event -> Relay ON

Test condition: event handler enabled, threshold set to 22.5 C. The simulator sends a DHT message with temperature 23.5 C and humidity 74.0%. Since 23.5 >= 22.5, the GUI reacts automatically by publishing a Relay ON command. The Relay state is visible as ON in both the main GUI and the simulator.

# 6. Live Test - Low Temperature Event -> Relay OFF

The simulator then sends a DHT message with temperature 21.5 C and humidity 76.0%. Because 21.5 < 22.5, the event-handler logic sends a Relay OFF command. The final Relay state is shown as OFF in both the main GUI and the simulator. The simulator log also shows the 21.5 C DHT messages used for this test.

# 7. Main Code Components

| File | Purpose |
| --- | --- |
| IoT_GUI_6233.py | Main PyQt5 application: connection settings, subscriber, publisher, Relay controls, event handler, message log and connection status. |
| CUBE_SIMULATOR_6233.py | Live test simulator that publishes DHT/Button events, receives Relay commands, and publishes Relay status. |
| requirements.txt | Python dependencies: paho-mqtt and PyQt5. |
| RUN_ASSIGNMENT3_6233.bat | Convenience launcher for the assignment files. |

# 8. Result Summary

GUI starts successfully and supports editable broker, port, running time, topics, Client ID, username and password.

MQTT connection status is shown directly in the interface.

Continuous subscription to the 6233 cube topic group is implemented.

DHT event handling was verified live through MQTT.

23.5 C with a 22.5 C threshold caused Relay ON.

21.5 C with the same threshold caused Relay OFF.

Manual publisher and manual Relay controls are included in addition to the automatic reaction.

A Button event-handler mode is also implemented and can toggle the Relay on each press.

Conclusion: The implemented PyQt5 application satisfies the Hands-On 6 GUI and MQTT requirements and demonstrates an automatic reaction to sensor data using a working event handler.

# Appendix – Full Source Code

Student: Itzhak Davidov | Last 4 digits: 6233

The complete source used for this assignment is included below and is also provided as separate files in the GitHub repository.

## IoT_GUI_6233.py

001  # Assignment 3 - IoT / Hands-On 6

002  # Student: Itzhak Davidov | Last 4 digits: 6233

003

004  import json

005  import random

006  import sys

007  from datetime import datetime

008

009  from PyQt5 import QtCore, QtGui, QtWidgets

010  import paho.mqtt.client as mqtt

011

012  STUDENT = "Itzhak Davidov"

013  LAST4 = "6233"

014

015  DEFAULT_BROKER = "broker.hivemq.com"

016  DEFAULT_PORT = 1883

017  COURSE_BROKER = "139.162.222.115"

018  COURSE_PORT = 80

019  COURSE_USERNAME = "MATZI"

020  COURSE_PASSWORD = "MATZI"

021

022  BASE_TOPIC = f"iot/{LAST4}/cubes"

023  DHT_TOPIC = f"{BASE_TOPIC}/dht/sts"

024  BUTTON_TOPIC = f"{BASE_TOPIC}/button/sts"

025  RELAY_CMD_TOPIC = f"{BASE_TOPIC}/relay/cmd"

026  RELAY_STATUS_TOPIC = f"{BASE_TOPIC}/relay/sts"

027  DEFAULT_SUB_TOPIC = f"{BASE_TOPIC}/#"

028

029

030  def make_client(client_id, clean_session=True):

031      """Create a paho client compatible with paho-mqtt 1.x and 2.x."""

032      try:

033          return mqtt.Client(

034              mqtt.CallbackAPIVersion.VERSION1,

035              client_id=client_id,

036              clean_session=clean_session,

037          )

038      except Exception:

039          return mqtt.Client(client_id=client_id, clean_session=clean_session)

040

041

042  class MqttSignals(QtCore.QObject):

043      connected = QtCore.pyqtSignal(int)

044      disconnected = QtCore.pyqtSignal(int)

045      message = QtCore.pyqtSignal(str, str)

046      log = QtCore.pyqtSignal(str)

047

048

049  class MainWindow(QtWidgets.QMainWindow):

050      def __init__(self):

051          super().__init__()

052          self.setWindowTitle(f"IoT Hands-On 6 - {STUDENT} ({LAST4})")

053          self.resize(1150, 760)

054

055          self.client = None

056          self.signals = MqttSignals()

057          self.signals.connected.connect(self.on_mqtt_connected)

058          self.signals.disconnected.connect(self.on_mqtt_disconnected)

059          self.signals.message.connect(self.on_mqtt_message)

060          self.signals.log.connect(self.append_log)

061

062          self.relay_state = False

063          self.last_event_command = None

064          self.running_timer = QtCore.QTimer(self)

065          self.running_timer.setSingleShot(True)

066          self.running_timer.timeout.connect(self.disconnect_mqtt)

067

068          self.build_ui()

069          self.apply_profile(0)

070

071      # ---------- UI ----------

072      def build_ui(self):

073          central = QtWidgets.QWidget()

074          self.setCentralWidget(central)

075          root = QtWidgets.QVBoxLayout(central)

076

077          title = QtWidgets.QLabel("Assignment 3 - MQTT GUI + Event Handler")

078          title.setAlignment(QtCore.Qt.AlignCenter)

079          title.setStyleSheet("font-size: 22px; font-weight: 700; padding: 8px;")

080          root.addWidget(title)

081

082          subtitle = QtWidgets.QLabel(

083              f"Student: {STUDENT} | Last 4 digits: {LAST4} | Topics: {BASE_TOPIC}/..."

084          )

085          subtitle.setAlignment(QtCore.Qt.AlignCenter)

086          subtitle.setStyleSheet("color: #555; padding-bottom: 8px;")

087          root.addWidget(subtitle)

088

089          # Connection group

090          connection = QtWidgets.QGroupBox("Connection / Session Settings")

091          grid = QtWidgets.QGridLayout(connection)

092

093          self.profile = QtWidgets.QComboBox()

094          self.profile.addItems(["HiveMQ Demo", "White Cube (course preset)"])

095          self.profile.currentIndexChanged.connect(self.apply_profile)

096

097          self.host = QtWidgets.QLineEdit()

098          self.port = QtWidgets.QSpinBox()

099          self.port.setRange(1, 65535)

100          self.runtime = QtWidgets.QSpinBox()

101          self.runtime.setRange(0, 86400)

102          self.runtime.setSuffix(" sec (0 = unlimited)")

103          self.runtime.setValue(0)

104          self.client_id = QtWidgets.QLineEdit(f"IOT_GUI_{LAST4}_{random.randint(1000, 9999)}")

105          self.username = QtWidgets.QLineEdit()

106          self.password = QtWidgets.QLineEdit()

107          self.password.setEchoMode(QtWidgets.QLineEdit.Password)

108          self.keepalive = QtWidgets.QSpinBox()

109          self.keepalive.setRange(10, 600)

110          self.keepalive.setValue(60)

111

112          self.connect_btn = QtWidgets.QPushButton("CONNECT")

113          self.connect_btn.clicked.connect(self.toggle_connection)

114          self.status = QtWidgets.QLabel("DISCONNECTED")

115          self.status.setAlignment(QtCore.Qt.AlignCenter)

116          self.set_status(False)

117

118          labels_widgets = [

119              ("Broker profile", self.profile),

120              ("Broker / Host", self.host),

121              ("Port", self.port),

122              ("Running time", self.runtime),

123              ("Client ID", self.client_id),

124              ("Username", self.username),

125              ("Password", self.password),

126              ("Keep Alive", self.keepalive),

127          ]

128          for i, (label, widget) in enumerate(labels_widgets):

129              row, col = divmod(i, 4)

130              base = col * 2

131              grid.addWidget(QtWidgets.QLabel(label), row, base)

132              grid.addWidget(widget, row, base + 1)

133

134          grid.addWidget(self.connect_btn, 2, 0, 1, 2)

135          grid.addWidget(QtWidgets.QLabel("Connection status"), 2, 2)

136          grid.addWidget(self.status, 2, 3, 1, 2)

137          root.addWidget(connection)

138

139          middle = QtWidgets.QHBoxLayout()

140          root.addLayout(middle, 1)

141

142          # Left: subscribe/publish

143          left = QtWidgets.QVBoxLayout()

144          middle.addLayout(left, 1)

145

146          sub_group = QtWidgets.QGroupBox("Continuous Subscriber")

147          sub_form = QtWidgets.QFormLayout(sub_group)

148          self.sub_topic = QtWidgets.QLineEdit(DEFAULT_SUB_TOPIC)

149          self.subscribe_btn = QtWidgets.QPushButton("SUBSCRIBE")

150          self.subscribe_btn.clicked.connect(self.subscribe_topic)

151          sub_form.addRow("Topic", self.sub_topic)

152          sub_form.addRow(self.subscribe_btn)

153          left.addWidget(sub_group)

154

155          pub_group = QtWidgets.QGroupBox("Publisher / Device Control")

156          pub_form = QtWidgets.QFormLayout(pub_group)

157          self.pub_topic = QtWidgets.QLineEdit(RELAY_CMD_TOPIC)

158          self.pub_message = QtWidgets.QLineEdit('{"value": 1}')

159          self.publish_btn = QtWidgets.QPushButton("PUBLISH")

160          self.publish_btn.clicked.connect(self.publish_manual)

161

162          relay_buttons = QtWidgets.QHBoxLayout()

163          self.relay_on_btn = QtWidgets.QPushButton("RELAY ON")

164          self.relay_off_btn = QtWidgets.QPushButton("RELAY OFF")

165          self.relay_on_btn.clicked.connect(lambda: self.set_relay(True, "manual control"))

166          self.relay_off_btn.clicked.connect(lambda: self.set_relay(False, "manual control"))

167          relay_buttons.addWidget(self.relay_on_btn)

168          relay_buttons.addWidget(self.relay_off_btn)

169          relay_widget = QtWidgets.QWidget()

170          relay_widget.setLayout(relay_buttons)

171

172          self.relay_indicator = QtWidgets.QLabel("UNKNOWN")

173          self.relay_indicator.setAlignment(QtCore.Qt.AlignCenter)

174          self.relay_indicator.setStyleSheet("font-size:18px;font-weight:700;background:#ddd;padding:6px;")

175

176          pub_form.addRow("Topic", self.pub_topic)

177          pub_form.addRow("Message", self.pub_message)

178          pub_form.addRow(self.publish_btn)

179          pub_form.addRow("Relay control", relay_widget)

180          pub_form.addRow("Relay status", self.relay_indicator)

181          left.addWidget(pub_group)

182

183          # Right: event handler

184          event_group = QtWidgets.QGroupBox("Event Handler / Automatic Reaction")

185          event_form = QtWidgets.QFormLayout(event_group)

186          self.event_enabled = QtWidgets.QCheckBox("Enable event handler")

187          self.event_enabled.setChecked(True)

188          self.event_mode = QtWidgets.QComboBox()

189          self.event_mode.addItems([

190              "DHT temperature -> Relay",

191              "Button press -> Relay toggle",

192          ])

193          self.threshold = QtWidgets.QDoubleSpinBox()

194          self.threshold.setRange(-20.0, 80.0)

195          self.threshold.setDecimals(1)

196          self.threshold.setValue(22.5)

197          self.threshold.setSuffix(" C")

198          self.event_status = QtWidgets.QLabel("Waiting for sensor event...")

199          self.event_status.setWordWrap(True)

200          self.event_status.setStyleSheet("font-weight:600;padding:8px;background:#f3f3f3;")

201

202          explanation = QtWidgets.QLabel(

203              "Reaction: in DHT mode, T >= threshold turns the relay ON; "

204              "T < threshold turns it OFF. In Button mode, each press toggles the relay."

205          )

206          explanation.setWordWrap(True)

207

208          event_form.addRow(self.event_enabled)

209          event_form.addRow("Event source", self.event_mode)

210          event_form.addRow("Temperature threshold", self.threshold)

211          event_form.addRow("Current reaction", self.event_status)

212          event_form.addRow(explanation)

213          middle.addWidget(event_group, 1)

214

215          # Log

216          log_group = QtWidgets.QGroupBox("MQTT Messages / Application Log")

217          log_layout = QtWidgets.QVBoxLayout(log_group)

218          self.log_box = QtWidgets.QPlainTextEdit()

219          self.log_box.setReadOnly(True)

220          self.log_box.setFont(QtGui.QFont("Consolas", 10))

221          clear_btn = QtWidgets.QPushButton("Clear log")

222          clear_btn.clicked.connect(self.log_box.clear)

223          log_layout.addWidget(self.log_box)

224          log_layout.addWidget(clear_btn)

225          root.addWidget(log_group, 2)

226

227      def apply_profile(self, index):

228          if index == 1:

229              self.host.setText(COURSE_BROKER)

230              self.port.setValue(COURSE_PORT)

231              self.username.setText(COURSE_USERNAME)

232              self.password.setText(COURSE_PASSWORD)

233          else:

234              self.host.setText(DEFAULT_BROKER)

235              self.port.setValue(DEFAULT_PORT)

236              self.username.clear()

237              self.password.clear()

238

239      def set_status(self, connected):

240          if connected:

241              self.status.setText("CONNECTED")

242              self.status.setStyleSheet(

243                  "background:#b8f5bf;color:#14551d;font-weight:800;padding:7px;border-radius:4px;"

244              )

245              self.connect_btn.setText("DISCONNECT")

246          else:

247              self.status.setText("DISCONNECTED")

248              self.status.setStyleSheet(

249                  "background:#ffd1d1;color:#7d1111;font-weight:800;padding:7px;border-radius:4px;"

250              )

251              self.connect_btn.setText("CONNECT")

252

253      # ---------- MQTT ----------

254      def toggle_connection(self):

255          if self.client is not None and self.client.is_connected():

256              self.disconnect_mqtt()

257          else:

258              self.connect_mqtt()

259

260      def connect_mqtt(self):

261          try:

262              cid = self.client_id.text().strip() or f"IOT_GUI_{LAST4}_{random.randint(1000,9999)}"

263              self.client = make_client(cid, clean_session=True)

264              self.client.on_connect = self._on_connect

265              self.client.on_disconnect = self._on_disconnect

266              self.client.on_message = self._on_message

267

268              user = self.username.text().strip()

269              if user:

270                  self.client.username_pw_set(user, self.password.text())

271

272              host = self.host.text().strip()

273              port = int(self.port.value())

274              keepalive = int(self.keepalive.value())

275              self.append_log(f"Connecting to {host}:{port} as {cid} ...")

276              self.client.connect(host, port, keepalive)

277              self.client.loop_start()

278          except Exception as exc:

279              self.append_log(f"CONNECT ERROR: {exc}")

280              QtWidgets.QMessageBox.critical(self, "MQTT connection error", str(exc))

281

282      def disconnect_mqtt(self):

283          self.running_timer.stop()

284          if self.client is not None:

285              try:

286                  self.client.loop_stop()

287                  self.client.disconnect()

288              except Exception:

289                  pass

290          self.set_status(False)

291          self.append_log("Disconnected")

292

293      def _on_connect(self, client, userdata, flags, rc):

294          self.signals.connected.emit(int(rc))

295

296      def _on_disconnect(self, client, userdata, rc, *args):

297          try:

298              code = int(rc)

299          except Exception:

300              code = 0

301          self.signals.disconnected.emit(code)

302

303      def _on_message(self, client, userdata, msg):

304          payload = msg.payload.decode("utf-8", "ignore")

305          self.signals.message.emit(msg.topic, payload)

306

307      @QtCore.pyqtSlot(int)

308      def on_mqtt_connected(self, rc):

309          if rc == 0:

310              self.set_status(True)

311              self.append_log("Connected OK")

312              self.subscribe_topic()

313              secs = int(self.runtime.value())

314              if secs > 0:

315                  self.running_timer.start(secs * 1000)

316                  self.append_log(f"Auto-disconnect scheduled after {secs} seconds")

317          else:

318              self.append_log(f"Connection failed, rc={rc}")

319              self.set_status(False)

320

321      @QtCore.pyqtSlot(int)

322      def on_mqtt_disconnected(self, rc):

323          self.set_status(False)

324          self.append_log(f"Disconnected (rc={rc})")

325

326      def subscribe_topic(self):

327          topic = self.sub_topic.text().strip()

328          if not topic:

329              return

330          if self.client is None or not self.client.is_connected():

331              self.append_log("Subscribe requested, but MQTT is not connected")

332              return

333          result, mid = self.client.subscribe(topic, qos=0)

334          self.append_log(f"SUBSCRIBE {topic} (result={result}, mid={mid})")

335

336      def publish_manual(self):

337          self.publish(self.pub_topic.text().strip(), self.pub_message.text())

338

339      def publish(self, topic, payload, qos=0, retain=False):

340          if self.client is None or not self.client.is_connected():

341              self.append_log("Publish requested, but MQTT is not connected")

342              return False

343          info = self.client.publish(topic, payload, qos=qos, retain=retain)

344          self.append_log(f"TX {topic}: {payload}")

345          return info.rc == mqtt.MQTT_ERR_SUCCESS

346

347      # ---------- Message handling / event handler ----------

348      @QtCore.pyqtSlot(str, str)

349      def on_mqtt_message(self, topic, payload):

350          self.append_log(f"RX {topic}: {payload}")

351

352          if topic == RELAY_STATUS_TOPIC:

353              self.handle_relay_status(payload)

354

355          if not self.event_enabled.isChecked():

356              return

357

358          mode = self.event_mode.currentText()

359          if mode.startswith("DHT") and topic == DHT_TOPIC:

360              self.handle_dht_event(payload)

361          elif mode.startswith("Button") and topic == BUTTON_TOPIC:

362              self.handle_button_event(payload)

363

364      def handle_relay_status(self, payload):

365          state = None

366          try:

367              data = json.loads(payload)

368              raw = str(data.get("relay", data.get("value", ""))).upper()

369          except Exception:

370              raw = payload.strip().upper()

371

372          if raw in {"1", "ON", "TRUE"}:

373              state = True

374          elif raw in {"0", "OFF", "FALSE"}:

375              state = False

376

377          if state is not None:

378              self.relay_state = state

379              text = "ON" if state else "OFF"

380              self.relay_indicator.setText(text)

381              bg = "#b8f5bf" if state else "#e6e6e6"

382              self.relay_indicator.setStyleSheet(

383                  f"font-size:18px;font-weight:700;background:{bg};padding:6px;"

384              )

385

386      def handle_dht_event(self, payload):

387          try:

388              data = json.loads(payload)

389              temp = float(data["temperature"])

390              humidity = float(data.get("humidity", 0.0))

391          except Exception as exc:

392              self.append_log(f"DHT parse error: {exc}")

393              return

394

395          threshold = float(self.threshold.value())

396          desired = temp >= threshold

397          action = "ON" if desired else "OFF"

398          self.event_status.setText(

399              f"DHT event: T={temp:.1f} C, H={humidity:.1f}% -> Relay {action} "

400              f"(threshold {threshold:.1f} C)"

401          )

402          self.append_log(

403              f"EVENT DHT: {temp:.1f} C {'>=' if desired else '<'} {threshold:.1f} C -> RELAY {action}"

404          )

405          # Avoid unnecessary repeated commands while readings stay on same side of threshold.

406          if self.last_event_command != desired:

407              self.set_relay(desired, "DHT event handler")

408              self.last_event_command = desired

409

410      def handle_button_event(self, payload):

411          pressed = False

412          try:

413              data = json.loads(payload)

414              pressed = int(data.get("value", 0)) == 1

415          except Exception:

416              pressed = payload.strip() in {"1", "ON", "TRUE", "PRESS"}

417

418          if pressed:

419              desired = not self.relay_state

420              self.event_status.setText(

421                  f"BUTTON event -> Relay {'ON' if desired else 'OFF'}"

422              )

423              self.append_log(

424                  f"EVENT BUTTON: press detected -> RELAY {'ON' if desired else 'OFF'}"

425              )

426              self.set_relay(desired, "button event handler")

427

428      def set_relay(self, on, reason="event handler"):

429          payload = json.dumps({"value": 1 if on else 0})

430          if self.publish(RELAY_CMD_TOPIC, payload):

431              self.append_log(f"REACTION ({reason}): relay command -> {'ON' if on else 'OFF'}")

432

433      def append_log(self, text):

434          stamp = datetime.now().strftime("%H:%M:%S")

435          self.log_box.appendPlainText(f"[{stamp}] {text}")

436          bar = self.log_box.verticalScrollBar()

437          bar.setValue(bar.maximum())

438

439      def closeEvent(self, event):

440          self.disconnect_mqtt()

441          event.accept()

442

443

444  def main():

445      app = QtWidgets.QApplication(sys.argv)

446      app.setStyle("Fusion")

447      window = MainWindow()

448      window.show()

449      sys.exit(app.exec_())

450

451

452  if __name__ == "__main__":

453      main()

## CUBE_SIMULATOR_6233.py

001  # Demo support for Assignment 3 - IoT / Hands-On 6

002  # Student: Itzhak Davidov | Last 4 digits: 6233

003

004  import json

005  import random

006  import sys

007  from PyQt5 import QtCore, QtWidgets

008  import paho.mqtt.client as mqtt

009

010  LAST4 = "6233"

011  BROKER = "broker.hivemq.com"

012  PORT = 1883

013  BASE = f"iot/{LAST4}/cubes"

014  DHT_TOPIC = f"{BASE}/dht/sts"

015  BUTTON_TOPIC = f"{BASE}/button/sts"

016  RELAY_CMD_TOPIC = f"{BASE}/relay/cmd"

017  RELAY_STATUS_TOPIC = f"{BASE}/relay/sts"

018

019

020  def make_client(client_id):

021      try:

022          return mqtt.Client(mqtt.CallbackAPIVersion.VERSION1, client_id=client_id, clean_session=True)

023      except Exception:

024          return mqtt.Client(client_id=client_id, clean_session=True)

025

026

027  class Signals(QtCore.QObject):

028      connected = QtCore.pyqtSignal(int)

029      relay_command = QtCore.pyqtSignal(str)

030

031

032  class Simulator(QtWidgets.QWidget):

033      def __init__(self):

034          super().__init__()

035          self.setWindowTitle("White Cube Demo Simulator - 6233")

036          self.resize(600, 360)

037          self.client = make_client(f"IOT_CUBE_SIM_{LAST4}_{random.randint(1000,9999)}")

038          self.client.on_connect = self._on_connect

039          self.client.on_message = self._on_message

040          self.signals = Signals()

041          self.signals.connected.connect(self.on_connected)

042          self.signals.relay_command.connect(self.on_relay_command)

043          self.relay_on = False

044          self.build_ui()

045

046      def build_ui(self):

047          layout = QtWidgets.QVBoxLayout(self)

048          title = QtWidgets.QLabel("MQTT Cube Simulator - DHT + Button + Relay")

049          title.setAlignment(QtCore.Qt.AlignCenter)

050          title.setStyleSheet("font-size:20px;font-weight:700;")

051          layout.addWidget(title)

052

053          self.status = QtWidgets.QLabel(f"Broker: {BROKER}:{PORT} | Disconnected")

054          self.status.setAlignment(QtCore.Qt.AlignCenter)

055          self.connect_btn = QtWidgets.QPushButton("CONNECT")

056          self.connect_btn.clicked.connect(self.connect_mqtt)

057          layout.addWidget(self.status)

058          layout.addWidget(self.connect_btn)

059

060          dht = QtWidgets.QGroupBox("DHT sensor events")

061          dht_layout = QtWidgets.QHBoxLayout(dht)

062          hot = QtWidgets.QPushButton("Send 23.5 C (Relay should turn ON)")

063          cold = QtWidgets.QPushButton("Send 21.5 C (Relay should turn OFF)")

064          hot.clicked.connect(lambda: self.send_dht(23.5, 74.0))

065          cold.clicked.connect(lambda: self.send_dht(21.5, 76.0))

066          dht_layout.addWidget(hot)

067          dht_layout.addWidget(cold)

068          layout.addWidget(dht)

069

070          btn_group = QtWidgets.QGroupBox("Button sensor event")

071          btn_layout = QtWidgets.QHBoxLayout(btn_group)

072          button = QtWidgets.QPushButton("PUSH BUTTON")

073          button.clicked.connect(self.send_button)

074          btn_layout.addWidget(button)

075          layout.addWidget(btn_group)

076

077          relay_group = QtWidgets.QGroupBox("Relay cube")

078          relay_layout = QtWidgets.QVBoxLayout(relay_group)

079          self.relay_label = QtWidgets.QLabel("OFF")

080          self.relay_label.setAlignment(QtCore.Qt.AlignCenter)

081          self.relay_label.setStyleSheet("font-size:30px;font-weight:800;background:#e6e6e6;padding:12px;")

082          relay_layout.addWidget(self.relay_label)

083          layout.addWidget(relay_group)

084

085          self.log = QtWidgets.QPlainTextEdit()

086          self.log.setReadOnly(True)

087          layout.addWidget(self.log)

088

089      def connect_mqtt(self):

090          self.status.setText(f"Broker: {BROKER}:{PORT} | Connecting...")

091          self.client.connect(BROKER, PORT, 60)

092          self.client.loop_start()

093

094      def _on_connect(self, client, userdata, flags, rc):

095          self.signals.connected.emit(int(rc))

096

097      def _on_message(self, client, userdata, msg):

098          self.signals.relay_command.emit(msg.payload.decode("utf-8", "ignore"))

099

100      def on_connected(self, rc):

101          if rc == 0:

102              self.status.setText(f"Broker: {BROKER}:{PORT} | CONNECTED")

103              self.connect_btn.setEnabled(False)

104              self.client.subscribe(RELAY_CMD_TOPIC, qos=0)

105              self.publish_relay_status()

106              self.log.appendPlainText(f"Subscribed: {RELAY_CMD_TOPIC}")

107          else:

108              self.status.setText(f"Connection failed rc={rc}")

109

110      def send_dht(self, temp, humidity):

111          payload = json.dumps({"temperature": temp, "humidity": humidity})

112          self.client.publish(DHT_TOPIC, payload, qos=0)

113          self.log.appendPlainText(f"DHT -> {DHT_TOPIC}: {payload}")

114

115      def send_button(self):

116          payload = json.dumps({"value": 1})

117          self.client.publish(BUTTON_TOPIC, payload, qos=0)

118          self.log.appendPlainText(f"BUTTON -> {BUTTON_TOPIC}: {payload}")

119

120      def on_relay_command(self, raw):

121          try:

122              data = json.loads(raw)

123              value = str(data.get("value", data.get("relay", ""))).upper()

124          except Exception:

125              value = raw.strip().upper()

126          if value in {"1", "ON", "TRUE"}:

127              self.relay_on = True

128          elif value in {"0", "OFF", "FALSE"}:

129              self.relay_on = False

130          elif value == "TOGGLE":

131              self.relay_on = not self.relay_on

132          else:

133              return

134          self.relay_label.setText("ON" if self.relay_on else "OFF")

135          self.relay_label.setStyleSheet(

136              "font-size:30px;font-weight:800;padding:12px;" +

137              ("background:#b8f5bf;" if self.relay_on else "background:#e6e6e6;")

138          )

139          self.log.appendPlainText(f"RELAY command received: {raw} -> {'ON' if self.relay_on else 'OFF'}")

140          self.publish_relay_status()

141

142      def publish_relay_status(self):

143          if self.client.is_connected():

144              payload = json.dumps({"relay": "ON" if self.relay_on else "OFF"})

145              self.client.publish(RELAY_STATUS_TOPIC, payload, qos=0, retain=True)

146

147      def closeEvent(self, event):

148          try:

149              self.client.loop_stop()

150              self.client.disconnect()

151          except Exception:

152              pass

153          event.accept()

154

155

156  if __name__ == "__main__":

157      app = QtWidgets.QApplication(sys.argv)

158      app.setStyle("Fusion")

159      w = Simulator()

160      w.show()

161      sys.exit(app.exec_())